Specifications for stage 1 (v0.1) :

## 1. Scope of Stage 1 (v0.1)

Goal of Stage 1:
Have a minimal but complete end-to-end system:

- C++ Bullet backend that simulates:

  - a single quadcopter rigid body

  - flat ground

- A clean C++ API around the sim core

- A Python binding exposing a Gym-style environment:

  - reset()

  - step(action)

  - returns a numerical observation and reward

- A simple “go to target point” task (no landing yet):

  - Start near origin.

  - Target point at a fixed 3D position.

  - Episode ends after fixed steps or if drone flies too far.

No RL training yet is required for v0.1, but the env must be ready for RL.

## 2. High-level objectives

By the end of Stage 1, you should have:

- Simulation core (C++ + Bullet)

  - Initialize Bullet world

  - Spawn ground + quadcopter body

  - Apply forces/torques to the drone

  - Step the simulation with a fixed dt (e.g. 1/240 s)

  - Read pose and velocities from Bullet

- Well-defined interfaces

  - C++ “SimCore” API with clear data types

  - Python Gym wrapper that calls into SimCore

- One simple task

  - Observation: drone state + relative position to target

  - Action: simple control vector (see below)

  - Reward + done logic defined

## 3. System architecture (Stage 1)

### 3.1 Modules

C++ side

- SimCore

  - Owns Bullet world and all Bullet objects.

  - Functions:

    - initialize(const WorldConfig&)

    - reset(const EpisodeConfig&)

    - step(const DroneAction&)

    - getObservation(DroneObservation&)

    - bool isDone()

    - float computeReward()

- DroneModel

  - Encapsulates the drone’s rigid body & parameters:

    - mass, inertia

    - position of rotors (for applying forces/torques)

Python side

- aerostack/envs/drone_nav_env.py
Implements a Gymnasium-like env:

  - reset(seed=None, options=None) -> obs, info

  - step(action) -> obs, reward, terminated, truncated, info

Internally, the Python env owns a handle/pointer to a SimCore instance exposed via bindings.

## 4. Physics & dynamics (Stage 1 simplifications)

To keep Stage 1 manageable:

- Use Bullet’s rigid body for the drone.

- Represent the drone as a single rigid body (no articulated parts).

- Ignore:

  - propeller aerodynamics

  - airflow, drag

  - detailed motor models

For v0.1:

- You can approximate the control action as body accelerations / forces instead of exact motor thrusts.

Later stages can refine this.

Suggested physical setup:

- Gravity: g = 9.81 m/s²

- Time step: dt = 1/240 s in Bullet, but your env may step at:

  - control_dt = 1/60 s (e.g. 4 Bullet substeps per control step)

## 5. Data structures (Stage 1)

### 5.1 World configuration

WorldConfig (C++ struct, conceptual):

- float gravity = -9.81f;

- float ground_z = 0.0f;

- Simulation parameters:

  - float sim_dt = 1.0f / 240.0f;

  - int substeps_per_control = 4;

For Stage 1, we can hardcode these or load from a small config file later.

### 5.2 Episode configuration

EpisodeConfig:

- Vector3 drone_initial_position (e.g. [0, 0, 1])

- Quaternion drone_initial_orientation (identity)

- Vector3 target_position (e.g. [2, 0, 1])

Optional (later): randomization ranges.

### 5.3 Drone action

Stage 1: keep it simple and high-level.

Option A (recommended to start): linear acceleration in body/world frame
DroneAction:

- Vector3 desired_accel in world frame

  - x, y: lateral acceleration

  - z: vertical acceleration offset from gravity

Option B (slightly closer to real quad): thrust + angular rates
DroneAction:

- float thrust (total collective)

- Vector3 body_rates (p, q, r)

You can pick one; for Stage 1 spec I’ll assume Option B (thrust + body rates), since it aligns better with later real control.

Range example:

- thrust ∈ [0, thrust_max]

- body_rates ∈ [-rate_max, rate_max] on each axis

### 5.4 Drone observation

DroneObservation should contain at least:

- Pose:

  - Vector3 position (world)

  - Quaternion orientation (world)

- Velocities:

  - Vector3 linear_velocity (world)

  - Vector3 angular_velocity (body or world)

- Mission info:

  - Vector3 target_position (world)

  - Or better: Vector3 position_error = target_position - position

For the Python env observation, flatten this into a 1D float array, e.g.:

[ pos_x, pos_y, pos_z,

vel_x, vel_y, vel_z,

roll, pitch, yaw,

ang_vel_x, ang_vel_y, ang_vel_z,

err_x, err_y, err_z ]

Angles can be derived from quaternion inside C++ before sending.

## 6. SimCore C++ API (Stage 1)

Conceptual interface (not exact code, just spec):

- void initialize(const WorldConfig& world_config);

  - Create Bullet world, ground, etc.

- void reset(const EpisodeConfig& episode_config);

  - Remove old bodies if needed.

  - Create drone rigid body at given pose.

  - Store target position.

- void step(const DroneAction& action);

  - Convert action into forces/torques on the drone.

  - Run Bullet stepping loop:

    - for substeps_per_control:

      - apply forces/torques

      - world->stepSimulation(sim_dt);

  - Clear forces.

- void getObservation(DroneObservation& obs) const;

  - Read position/orientation from Bullet.

  - Compute velocities.

  - Fill in error to target.

- float computeReward() const;

  - (Defined below)

- bool isDone() const;

  - (Defined below)

SimCore also keeps internal counters:

- int step_count

- int max_steps_per_episode

## 7. Reward and termination (Stage 1 task: go-to-point)

### 7.1 Task description

- Task: reach and stay near target_position.

- Start: near origin at fixed height.

- No landing yet; just position control in 3D.

### 7.2 Reward design (Stage 1, simple)

A simple dense reward:

- Let d = ||position - target_position||

- Reward per step:

r = -d

- alpha * ||linear_velocity||^2

- beta  * ||angular_velocity||^2

- gamma * ||body_rates||^2  (action penalty, optional)

Where alpha, beta, gamma are small positive constants.

Optional positive bonus if very close to target:

- If d < d_success_threshold, add a small constant bonus per step, e.g. +0.1.

### 7.3 Termination conditions

Episode ends if:

- Step limit reached:

  - step_count >= max_steps_per_episode.

- Drone “escapes” bounds:

  - ||position|| > R_max or pos_z < z_min (below ground too far).

- (Later) if stable near target for some time.

At Stage 1, a simple done condition:

- done = (step_count >= max_steps) || (pos_z < 0.0f) || (||position|| > R_max)

You can treat going underground as a crash.

## 8. Python Gym environment spec

File: aerostack/envs/drone_nav_env.py

Spaces (conceptual):

- observation_space: Box(low=-inf, high=inf, shape=(N,), dtype=float32)

- action_space: Box(low=[thrust_min, -rate_max, -rate_max, -rate_max], high=[thrust_max, rate_max, rate_max, rate_max], dtype=float32)

Core methods:

- __init__(...)

  - Loads world config, default episode config.

  - Instantiates a SimCore object via bindings.

- reset(seed=None, options=None)

  - Optionally randomize initial drone pose and/or target pos (later).

  - Calls sim_core.reset(episode_config).

  - Calls sim_core.getObservation() → builds obs array.

  - Resets internal step counter.

  - Returns (obs, info).

- step(action)

  - Validates/clips action w.r.t action_space.

  - Calls sim_core.step(action).

  - Calls:

    - sim_core.getObservation()

    - reward = sim_core.computeReward()

    - done = sim_core.isDone()

  - Builds info dict (for Stage 1: maybe empty or distance to target).

  - Uses Gymnasium’s terminated / truncated split:

    - terminated = crash / leaving bounds

    - truncated = time limit reached

  - Returns (obs, reward, terminated, truncated, info).

## 9. Minimal testing / acceptance criteria for Stage 1

Stage 1 is done and successful when:

- You can run a small Python script that:

  - Instantiates DroneNavEnv.

  - Calls reset().

  - Loops for, say, 200 steps with random or zero actions.

  - Plots or prints positions over time.

- The drone:

  - Appears in the world at the correct initial position.

  - Falls under gravity if thrust is low.

  - Moves in plausible ways when you send actions.

  - Provides consistent observations and rewards.

- The environment:

  - Produces no crashes or memory issues for many episodes.

  - Has stable step timing (no weird dt fluctuations).

  - Is compatible with a basic Gym-style RL loop (even if you don’t train yet).